## Brief Embedded Linux Boot Steps

- Power-On Reset (POR)

  - Hardware powers up, CPU starts executing from internal ROM.

- ROM Bootloader (Stage 0)

  - Detects & Initialize boot source (e.g., SPI-NAND/NOR).

  - Loads Stage 1 bootloader into SRAM.

- Stage 1 Bootloader (SPL / Preloader)

  - Initializes DDR RAM and basic peripherals.

  - Loads Stage 2 bootloader into RAM.

- Stage 2 Bootloader (U-Boot / Breed)

  - Initializes console UART, flash partition, and environment.

  - Loads Linux kernel, DTB, and initramfs or rootfs.

- Kernel Boot

  - Kernel decompresses and starts execution.

  - Initializes CPU, memory, devices, and drivers.

- Root Filesystem Mount

  - Mounts rootfs from flash, SD card, or network.

  - Starts init process.

- Init System Execution

  - Runs startup scripts and services.

  - Configures network, starts daemons.

- System Ready

  - User interfaces (CLI, web UI) become available.

  - Application layer starts running.

## Detailed

## 🧠 Embedded Linux Boot Process – Detailed Breakdown

### 🔹 1. Power-On Reset (POR)

- Hardware powers up.

- CPU starts executing from a fixed address in internal ROM.

- Boot source (e.g., SPI-NAND, NOR, eMMC) is detected.

### 🔹 2. ROM Bootloader (Stage 0)

- Executes from internal ROM of the SoC.

- Initializes minimal peripherals (SPI, UART).

- Loads Stage 1 bootloader (e.g., SPL, Preloader) from flash into SRAM.

### 🔹 3. Stage 1 Bootloader (SPL / Preloader)

- Initializes:

  - DDR RAM (timing, voltage)

  - Clock system

  - Basic I/O (UART, GPIO)

- Loads Stage 2 bootloader (U-Boot or equivalent) into RAM.

### 🔹 4. Stage 2 Bootloader (U-Boot / Barebox / GRUB)

- Full-featured bootloader.

- Initializes:

  - Console (UART)

  - Flash partitions (MTD)

  - Environment variables

  - Network (TFTP, DHCP if needed)

- Loads:

  - Linux kernel (zImage, uImage)

  - Device Tree Blob (.dtb)

  - Initial RAM filesystem (initramfs) or rootfs

### 🔹 5. Kernel Decompression & Execution

- Kernel decompresses (LZMA, GZIP, etc.).

- Parses boot arguments from bootloader.

- Initializes:

  - Memory zones

  - CPU cores and scheduler

  - Interrupt controller

  - Device drivers (SPI, I2C, USB, Ethernet)

  - Filesystem support (UBIFS, SquashFS, ext4)

### 🔹 6. Root Filesystem Mount

- Mounts root filesystem from:

  - Flash (MTD block)

  - UBI volume

  - SD card/eMMC

  - Network (NFS)

- Starts init process (PID 1).

### 🔹 7. Init System (SysVinit / systemd / procd)

- Executes startup scripts:

  - Mounts /proc, /sys, /dev

  - Starts daemons (network, SSH, web UI)

  - Configures interfaces (eth0, wlan0)

- Brings system to multi-user mode.

### 🔹 8. Application Layer

- Services like:

  - LuCI (OpenWRT web interface)

  - Dropbear/SSH

  - Firewall (iptables/nftables)

  - Custom embedded apps

## 📦 Flash Partition Layout (Typical)

[ bootloader ] → [ factory ] → [ kernel ] → [ rootfs ] → [ overlay ]

- bootloader: U-Boot or Breed

- factory: MAC address, WiFi calibration

- kernel: Linux image

- rootfs: SquashFS or UBIFS

- overlay: Writable partition for config changes